DELTA PSU PMBUS Software User Manual

| Date | Version | Description / Modifications | Author |
| --- | --- | --- | --- |
| 2018-08-16 | S00 | Draft frame | Linchun.Li |
| 2018-11-09 | S01 | Modify software name and version in 1.1. Add model information description of multi-channel PSU in 3.1 | Linchun.Li |
| 2023-06-30 | S02 | Add operating log description in 3.5 | Linchun.Li |
| 2025-05-22 | S03 | Add new product series to Chapter 2.1 | Linchun.Li |

1	Overview	4

1.1	Introduction	4

1.2	Communication Structure	4

1.3	Acronyms and Definitions	4

2	General Information	4

2.1	Supported PSU Series	4

2.2	Installation	5

2.3	Starting Utility	5

2.4	User Management	6

2.5	Address Setting	8

2.6	Port Setting	8

3	PSU Monitor	8

3.1	Power Supply Information	9

3.2	Channel Status	10

3.3	Running Status	11

3.4	Output Information	11

3.5	Operating Log	12

4	PSU Setting	13

4.1	Settings	13

4.2	Module Settings	15

Overview

Introduction

The DELTA PSU Software is software package, installed on windows based computer, designed for user to configure parameters or check the operate status of the PSU with PMBUS interface.

A Delta PMBUS adapter with USB or RS232 interface should be used between the PSU and the PC, providing a complete interface between the PSU device and the I2C bus with a simple command set which enables the highest levels of configuration flexibility, detailed please see the DELTA PSU Communication Interface Adapter Specification V05.

This document describes the PMBUS software function and its utilized methods. This user manual must be read and fully understood before using. This Utility is supposed to be used by the authorized user or service personnel.

All the pictures and figures in this manual are for illustration. The exact data shown in the figures will vary from model to model and be subject to real-time change.

This user manual version is corresponded to Delta Configurable Power Software V02.17 and above.

Communication Structure

The picture below is the communication structure of the whole power supply system.

Acronyms and Definitions

ACK: ACKnowedge. The response from a receiving unit indicating that it has received a byte.

NACK: Not ACKnowledge. The response from a receiving unit that it has received invalid data.

LSB: Least significant bit

MSB: Most significant bit

CMD: command

CAN: Controller Area Network

CRC: Cyclical Redundancy Checking

I2C: Inter－Integrated Circuit

General Information

Supported PSU Series

| PSU Type | Name | Description |
| --- | --- | --- |
| Configurable PSU | MEG-1K2 series | 4 channel configurable PSU |
| Configurable PSU | MEG-2K1 series | 6 channel configurable PSU |
| Configurable PSU | MEG-3K0 series | 9 channel configurable PSU |
| Standard PSU | MEB-1K2/2K5 series | 1 output standard PSU |
| Standard PSU | MEU-600C series | 1 output standard PSU |
| Standard PSU | PMS series | 1 output standard PSU |

Installation

The DELTA PSU PMBUS Software is available on following operating systems:

2000 SP3/ XP SP2/ 2003 Server/ 2008 Server/ Windows Vista Business/ Windows Vista Enterprise/ Windows Vista Home Basic/ Windows Vista Home Premium/ Windows Vista Starter/ Windows Vista Ultimate/ Windows 7/ Windows 8 / Windows 10// Windows 11

Click PSU_Software.exe to start the GUI.

The DELTA PSU PMBUS Software runs on Microsoft .NET Framework 2.0. Therefore, the .NET Framework 2.0 will be installed if it is not installed on the user’s computer before.

Starting Utility

Once DELTA PSU PMBUS Software has been successfully downloaded, it can be started by selecting the icon  from the folder.

Once DELTA PSU PMBUS Software has been started, a Login page will pop up. Enter the login account name and password. Click “Login”. Note: the password is case-sensitive.

Default admin account is “Admin”, and the password is “Admin”.

User Management

Administrator can create/ delete users and reset the password by select the “Account” item and enter the Manage Account page after set the origin account and password

“Account”enter origin account and password ”Login”:

“Create Account”: create new Account, only English letters or numbers could be used, and the length should be 4~20.

“Edit Account”: change current password, only English letters or numbers could be used, and the length should be 4~20.

“Delete Account”: delete current password

Address Setting

Before connect to the PSU, the address of the PSU should be set on the GUI

Default mode: the address is fixed to 0x57, could not be changed.

GPIO mode: the address is defined by the user (first 4 bits) and the address switch (last 3 bits) on the adapter, after connected successfully, the current address would be displayed on the address table.

Change mode: the whole address is defined by the user.

Port Setting

The user should choose the COM ports before connect:

DELTA PSU PMBUS Software will auto-detect the COM ports that the user’s computer is using, and show in the combo box. After selecting a COM port, click “Connect”.

If there is no COM port detected when the PSU connected, the user should check the driver install status, the procedure is shown as below:

Step1: START—Windows System—Control Panel

Step 2: press device manager

Step 3: connect the USB line to the computor, there will be a new device under the “COM and LPT” item

Step 4: right click and choose “update the driver”

Step 5: choose “Browse the computer to find the driver”

Step 6: find the “usb-rs232” folder in the DELTA PSU PMBUS Software download folder and press “next”, wait for the driver install finished, if new COM number not generate, repeat step 3~6 again

PSU Monitor

This section is for user to check the PSU status, including the model name, input/output parameters, temperature and protection information. The picture below is the overview of the monitor GUI:

Power Supply Information

In the Power Supply information area, the S/N and model name is displayed. Before connection, the user should make sure there is the command list (.xml) and the picture (.jpg or .png) in the software folder, the name of the file is matched with the model name that the PSU reported, or the connection would be fail. For multi-channel power supply, there should be MEG-xxx.xml in the folder.

Channel Status

The operate status is displayed in this area. When the PSU operate normally, the status would be OK, or the error type would be displayed in corresponding channel.

In multi-channel PSU, if there are not any modules connected, the channel status would be empty.

Press the status, the user could press the error to find detailed information. The information could be refreshed by click the “READ FAULT” button, the fault report could be cleared by press “CLEAR FAULT”.

Running Status

The output power, temperature, input voltage and fan speed would be displayed in this area. The status is the whole PSU status.

Output Information

This area is displayed the output voltage, current and temperature of the detailed channel. The minimum current, maximum current and the OCP fault current would be displayed in the current display area to indicate the load status.

In multi-channel PSU, the connected module name would be displayed.

Operating Log

This GUI could store up to 5 min output data including output voltage, current, temperature and fan speed, the user could press corresponding data area to check the operating status

The data curve is shown as below:

PSU Setting

This section is for user to set the output parameters, change the operate mode, on/off the PSU and check the module information of each module.

Settings

Press the “Settings” in the output information area, the following page will pop up:

Remote control area:

Local control: the output voltage is defined by the VR in the PSU, in this mode, the CC_CV_MODE area is disabled.

Remote control: the output voltage is defined by the GUI.

CC_CV mode area:

CV mode: change the output voltage by the point at left.

CC mode: change the output current in the box (some product don’t support this mode, need to check the specification first)

Operation area:

On/Off the corresponding modules.

Module Settings

This function is for user to check the module information, protection set point and derating point.

Press the “ModuleSettings” in the output information area, the following page will pop up:

In Basic Information tag, the module name, module S/N and module FW version is shown, these information is read only.

In protection area, the Vout_OV_Fault_Limit, Iout_OC_Fault_Limit, OT_Fault_Limit and UT_Fault_Limit could be checked, all the information is read only.

In Derating area, the VOUT_MAX, VOUT_MIN, IOUT_MAX, IOUT_MIN could be checked, only IOUT_MAX could be set, others are read only.